UNMANNED AERIAL SYSTEMS

Skills Learned: Airfoil Theory, Aerodynamics &

Structural Design, Stability and Control, Sensors

NASWIHA NOWSHAD

B240077EC

# Minor Course: Unmanned Aerial Systems

# Airfoil Theory

# 1. Airfoils Selected for Analysis

The following airfoils were selected for aerodynamic comparison in the parent–child docking drone project:

- NACA 0012 airfoil

- NACA 2412 airfoil

- NACA 4412 airfoil

- S1223 airfoil

2. Software and Analysis Method

Airfoil analysis was performed using:

- XFLR5

The software was used for:

- Airfoil generation/import

- Polar analysis

- Lift and drag evaluation

- Aerodynamic efficiency comparison

- Pressure coefficient analysis

# 3. Airfoil Modeling Procedure

### Step 1 — Airfoil Generation

NACA airfoils were generated using the built-in NACA foil generator in XFLR5.

S1223 airfoil was imported using DAT coordinate files.

### Step 2 — Polar Definition

Type 1 fixed-speed polar analysis was selected.

Parameters used:

| Parameter | Value |
| --- | --- |
| Reynolds Number | 200000 |
| Mach Number | 0 |
| Ncrit | 9 |
| Alpha Range | -5° to 15° |
| Increment | 0.5° |

### Step 3 — Aerodynamic Analysis

Aerodynamic analysis was executed for all selected airfoils under identical conditions.

### Step 4 — Graph Generation

The following graphs were generated:

- Cl vs Alpha

- Cd vs Alpha

- Cl/Cd vs Alpha

- Pressure coefficient distribution (Cp)

# 4. Lift Coefficient Analysis

# Lift coefficient variation with angle of attack was analyzed for all selected airfoils.

# Observations

- Lift coefficient increased approximately linearly with increasing angle of attack before stall.

- NACA 4412 generated higher lift compared to NACA 0012.

- S1223 produced the highest lift characteristics at low Reynolds numbers.

- NACA 0012 showed smoother and more symmetric behavior.

## Comparison

| Airfoil | Lift Characteristics |
| --- | --- |
| NACA 0012 | Moderate lift |
| NACA 2412 | Balanced lift |
| NACA 4412 | High lift |
| S1223 | Very high lift |

# 5. Drag Coefficient Analysis

Drag coefficient variation with angle of attack was studied using polar analysis.

## Observations

- Drag increased gradually with angle of attack.

- Significant drag rise occurred after stall angle.

- NACA 0012 showed lower drag at moderate angles.

- S1223 produced higher drag at larger angles due to stronger camber.

## Comparison

| Airfoil | Drag Characteristics |
| --- | --- |
| NACA 0012 | Low drag |
| NACA 2412 | Moderate drag |
| NACA 4412 | Slightly higher drag |
| S1223 | Higher drag at high alpha |

# 6. Aerodynamic Efficiency Analysis

Lift-to-drag ratio analysis was conducted using Cl/Cd graphs.

## Observations

- Maximum aerodynamic efficiency occurred at moderate angles of attack.

- NACA 2412 showed stable and balanced efficiency characteristics.

- S1223 exhibited excellent low-speed efficiency for UAV operation.

- NACA 0012 demonstrated predictable performance.

## Comparison

| Airfoil | Efficiency |
| --- | --- |
| NACA 0012 | Stable |
| NACA 2412 | Balanced |
| NACA 4412 | High lift efficiency |
| S1223 | Excellent low-speed efficiency |

# 7. Pressure Distribution Analysis

Pressure coefficient (Cp) distribution was analyzed for selected operating points.

## Observations

- Lower pressure regions were observed on the upper airfoil surface.

- Higher pressure regions were observed on the lower surface.

- Strong suction peaks were observed near the leading edge.

- Pressure difference between upper and lower surfaces contributed to lift generation.

8. Stall Behavior Analysis

## Observations

- Stall behavior was identified from sudden reduction in lift coefficient.

- NACA 4412 and S1223 showed higher lift before stall.

- NACA 0012 demonstrated smoother stall characteristics.

9. Final Comparison of Airfoils

| Airfoil | Lift | Drag | Stability | UAV Suitability |
| --- | --- | --- | --- | --- |
| NACA 0012 | Moderate | Low | Excellent | Moderate |
| NACA 2412 | Balanced | Moderate | Good | Good |
| NACA 4412 | High | Moderate | Moderate | Very Good |
| S1223 | Very High | Higher | Moderate | Excellent |

# 10. Conclusion

Airfoil analysis was successfully performed using XFLR5 for low-Reynolds-number UAV conditions relevant to the parent–child docking drone system.

Comparative aerodynamic analysis showed that:

- NACA 0012 provided stable and symmetric aerodynamic behavior.

- NACA 2412 demonstrated balanced lift and efficiency characteristics.

- NACA 4412 generated higher lift suitable for payload carrying applications.

- S1223 exhibited superior low-speed lift performance and UAV efficiency.

Based on the conducted analysis, cambered airfoils such as NACA 4412 and S1223 were found more suitable for low-speed UAV operations requiring improved lift generation and aerodynamic efficiency.

# Aerodynamics and Structural Design

# 1. Software and Tools Used

The following software tools were used for aerodynamic and structural investigations:

- Fusion 360

- ANSYS Fluent

- ANSYS Workbench

2. CAD Modeling of Drone Structure

A simplified parent–child drone geometry was developed using Fusion 360.

The CAD model included:

- Central drone body

- Motor arms

- Propeller regions

- Docking section

- Child drone accommodation region

The geometry was simplified to improve CFD meshing and reduce computational complexity.

# 3. Aerodynamic CFD Analysis

Computational Fluid Dynamics (CFD) analysis was conducted in ANSYS Fluent to study airflow behavior around the drone structure.

## CFD Procedure Performed

### Step 1 — Geometry Import

The CAD model was exported as a STEP file and imported into ANSYS Workbench.

### Step 2 — Fluid Domain Creation

An external air enclosure was generated around the drone body to simulate surrounding airflow conditions.

### Step 3 — Meshing

A computational mesh was generated with finer mesh regions near:

- Drone body

- Arm junctions

- Docking interface

### Step 4 — Boundary Conditions

The following conditions were applied:

| Parameter | Value |
| --- | --- |
| Inlet Velocity | 10 m/s |
| Outlet | Pressure outlet |
| Solver Type | Pressure-based |
| Flow Type | Steady-state |
| Turbulence Model | k-ω SST |

### Step 5 — CFD Simulation

The solver was executed for multiple iterations until convergence.

### Step 6 — Result Visualization

The following outputs were generated:

- Velocity contours

- Pressure contours

- Streamlines

- Turbulence regions

# 4. Aerodynamic Observations

## Velocity Contours

### Observations

- Higher airflow velocity regions were observed around arm edges and docking regions.

- Wake formation occurred behind the drone body.

- Flow acceleration was observed near narrow structural regions.

## Pressure Distribution

### Observations

- High-pressure regions formed at frontal surfaces.

- Low-pressure zones appeared along upper body surfaces.

- Pressure imbalance contributed to aerodynamic loading.

## Streamline Analysis

### Observations

- Streamline separation occurred near sharp geometry transitions.

- Disturbed flow regions were identified behind the docking interface.

- Smooth body curvature improved airflow continuity.

## Turbulence Analysis

### Observations

- Increased turbulence intensity was observed near docking structures.

- Rotor interaction regions produced disturbed airflow patterns.

# 5. Aerodynamic Comparisons

## Comparison Between Structural Regions

| Region | Aerodynamic Behavior |
| --- | --- |
| Rounded body surfaces | Smooth airflow |
| Sharp edges | Flow separation |
| Docking interface | Local turbulence |
| Arm junctions | Velocity concentration |

# 6. Structural Design Analysis

Structural analysis was performed to evaluate:

- Stress concentration

- Structural deformation

- Load-bearing capability

# Structural Analysis Procedure

### Step 1 — Material Assignment

Lightweight structural materials were considered for UAV applications.

Example:

- Carbon fiber composite

- Aluminum alloy

### Step 2 — Boundary Conditions

Fixed supports were applied near:

- Central frame regions

Loads were applied at:

- Motor mounting regions

- Docking interface locations

### Step 3 — Static Structural Simulation

Static structural analysis was executed in ANSYS Workbench.

### Step 4 — Output Generation

The following results were obtained:

- Equivalent stress plots

- Total deformation plots

- Structural displacement contours

# 7. Structural Observations

## Stress Distribution

### Observations

- Maximum stress concentration occurred near motor arm joints.

- Docking interface regions experienced localized stress loading.

- Stress remained within acceptable structural limits.

## Deformation Analysis

### Observations

- Small structural deformation was observed under loading conditions.

- Maximum deformation occurred near unsupported arm extremities.

- Central body region remained structurally stable.

# 8. Structural Comparisons

| Structural Region | Stress Level | Deformation |
| --- | --- | --- |
| Motor arm joints | High | Moderate |
| Central body | Low | Low |
| Docking interface | Moderate | Moderate |
| Outer arm sections | Moderate | Higher |

# 9. Design Improvements Considered

The following improvements were identified during analysis:

- Increased reinforcement near arm joints

- Smoother aerodynamic transitions

- Reduced sharp edges near docking interface

- Improved load distribution across frame

# 10. Conclusion

Aerodynamic and structural investigations were successfully conducted for the parent–child docking drone system using Fusion 360 and ANSYS tools.

CFD analysis identified:

- Wake formation

- Turbulence regions

- Pressure concentration areas

- Airflow disturbances near docking regions

Structural analysis identified:

- Stress concentration near motor mounts

- Controlled structural deformation

- Stable central body behavior

The combined aerodynamic and structural studies contributed to optimization of:

- UAV stability

- Structural integrity

- Docking efficiency

- Aerodynamic performance

The performed simulations validated the feasibility of the proposed drone configuration for low-speed UAV docking operations.

# Stability and Control

# 1. Stability Parameters Studied

The following stability parameters of UAV systems were studied:

- Roll stability

- Pitch stability

- Yaw stability

- Hover stability

- Dynamic response characteristics

- Disturbance response

- Docking alignment stability

# 2. Roll Stability

The effect of roll motion on quadcopter balance and lateral stability was studied.

The relationship between:

- Motor thrust imbalance

- Roll angle variation

- Lateral motion

was analyzed.

## Observations

- Small thrust variations between opposite motors caused roll motion.

- Excessive roll angle reduced flight stability.

- Oscillatory roll motion affected docking alignment accuracy.

- Balanced thrust distribution improved lateral stability.

# 3. Pitch Stability

Pitch dynamics during forward and backward motion were studied.

The effect of:

- Center of gravity location

- Thrust variation

- Drone geometry

on pitch stability was analyzed.

## Observations

- Forward shift in center of gravity increased pitching tendency.

- Uneven thrust distribution produced unstable pitch motion.

- Stable pitch behavior improved docking precision.

- Aerodynamic disturbances affected pitch response during hover.

# 4. Yaw Stability

Yaw motion and directional stability of the drone were studied.

The influence of:

- Rotor torque

- Counter-rotating propellers

- Rotational balance

was investigated.

## Observations

- Rotor torque imbalance caused yaw drift.

- Counter-rotating propellers improved directional stability.

- Sudden yaw motion affected docking orientation.

- Controlled yaw response improved alignment accuracy.

# 5. Hover Stability

Hovering conditions were studied to understand stability during stationary flight and docking operations.

The influence of:

- Wind disturbances

- Rotor wash interaction

- Small positional deviations

was analyzed.

## Observations

- Hover instability increased docking difficulty.

- Rotor interaction generated local airflow disturbances.

- Small positional oscillations reduced docking precision.

- Stable hovering improved docking feasibility.

# 6. Dynamic Response Analysis

The transient response of the drone under disturbance conditions was studied.

The following behaviors were analyzed:

- Oscillatory response

- Settling behavior

- Response time

- Overshoot characteristics

## Observations

- Sudden disturbances caused temporary oscillatory motion.

- Excessive oscillations reduced flight stability.

- Faster settling improved system controllability.

- Reduced overshoot improved stabilization behavior.

# 7. Center of Gravity Analysis

The effect of center of gravity position on flight stability was studied.

Different payload and docking configurations were considered.

## Observations

- Off-center payload distribution reduced stability.

- Centralized mass distribution improved balance.

- Docking mechanisms influenced center of gravity location.

- Stable CG positioning improved hover performance.

# 8. Docking Stability

The stability requirements during parent–child docking operations were investigated.

The influence of:

- Relative alignment

- Hover precision

- Orientation stability

was analyzed.

## Observations

- Small angular deviations affected docking accuracy.

- Stable roll and pitch behavior were essential during docking.

- Reduced oscillations improved alignment feasibility.

- Hover stabilization was critical for successful docking.

# 9. Comparative Stability Analysis

| Parameter | Stable Configuration | Unstable Configuration |
| --- | --- | --- |
| Roll Motion | Minimal oscillation | Large oscillation |
| Pitch Response | Smooth response | Sudden deviations |
| Yaw Stability | Controlled rotation | Drift motion |
| Hovering | Stable | Position fluctuations |
| Docking Alignment | Accurate | Misalignment |

# 10. Key Findings

The conducted study identified that:

- Stable thrust distribution is essential for UAV balance.

- Hover precision directly affects docking success.

- Center of gravity plays a major role in stability.

- Roll and pitch oscillations significantly influence docking accuracy.

- Directional control is important for autonomous alignment.

# 11. Conclusion

The stability and control characteristics of the parent–child docking drone system were studied with emphasis on roll, pitch, yaw, and hover behavior.

The conducted research showed that:

- Stable thrust balance improves flight control.

- Controlled attitude response improves docking precision.

- Hover stability is critical for autonomous docking operations.

- Proper center of gravity distribution enhances UAV stability.

- Reduced oscillatory behavior improves overall controllability.

The study validated the importance of stable attitude control and dynamic balance for reliable parent–child docking drone operations.

# Sensors

# 1. Sensors Studied for the Drone System

The following sensors relevant to the parent–child docking drone system were studied:

- Ultrasonic sensor

- Infrared (IR) sensor

- IMU (Accelerometer + Gyroscope)

- GPS module

- Barometer

- Magnetometer

- Camera/Vision sensor

The study focused on:

- Navigation

- Stabilization

- Position estimation

- Hover control

- Docking assistance

- Relative alignment

# 2. Ultrasonic Sensor Study

Ultrasonic sensing was studied for:

- Obstacle detection

- Distance measurement

- Docking proximity estimation

The working principle based on ultrasonic pulse transmission and echo reception was analyzed.

## Observations

- Accurate short-range distance measurement was obtained.

- Sensor performance decreased at larger distances.

- Surface irregularities affected echo response.

- Useful for close-range docking alignment.

# 3. Infrared (IR) Sensor Study

Infrared sensing was studied for:

- Short-range object detection

- Docking position sensing

- Relative positioning assistance

## Observations

- Fast response time was observed.

- Detection accuracy reduced under strong ambient lighting.

- Effective for short-range alignment applications.

- Useful for close docking guidance.

# 4. IMU Sensor Study

The Inertial Measurement Unit (IMU), consisting of accelerometer and gyroscope sensors, was studied for:

- Orientation sensing

- Attitude stabilization

- Motion tracking

The influence of IMU feedback on flight stability and hover control was analyzed.

## Observations

- Gyroscope measured angular motion effectively.

- Accelerometer detected linear acceleration and tilt.

- Combined sensor fusion improved orientation estimation.

- IMU data was essential for stable flight control.

# 5. GPS Sensor Study

GPS-based positioning was studied for:

- Global navigation

- Outdoor positioning

- Path tracking

## Observations

- GPS enabled long-range position estimation.

- Accuracy reduced in indoor or obstructed environments.

- Useful for large-scale navigation.

- Limited precision for close docking operations.

# 6. Barometer Study

Barometric sensing was studied for:

- Altitude estimation

- Hover stabilization

- Vertical position control

## Observations

- Atmospheric pressure variation enabled altitude estimation.

- Small pressure fluctuations affected accuracy.

- Useful for maintaining stable hover altitude.

- Improved vertical positioning capability.

# 7. Magnetometer Study

Magnetometer sensing was studied for directional orientation and heading estimation.

## Observations

- Magnetic field sensing improved directional awareness.

- Nearby electronic interference affected readings.

- Useful for yaw orientation stabilization.

- Assisted in navigation alignment.

# 8. Vision Sensor / Camera Study

Camera-based sensing was studied for:

- Visual docking assistance

- Object recognition

- Relative positioning

The role of vision sensing in autonomous docking and alignment was investigated.

## Observations

- Vision sensing improved alignment capability.

- Lighting conditions affected detection quality.

- Image processing increased computational requirement.

- Useful for autonomous docking guidance.

# 9. Sensor Interfacing Study

Sensor interfacing methods and communication protocols were studied for reliable integration with flight controllers and embedded systems.

The following interfaces were analyzed:

- I2C communication

- SPI communication

- UART communication

## Observations

- I2C communication simplified multi-sensor interfacing.

- UART communication was suitable for GPS modules.

- SPI communication provided faster data transfer rates.

- Proper sensor interfacing improved data reliability.

# 10. Sensor Fusion Study

The integration of multiple sensors for improved drone performance was studied.

The following combinations were investigated:

- IMU + GPS

- IMU + Barometer

- Camera + Ultrasonic sensor

Basic concepts of sensor fusion and combined sensing were analyzed.

## Observations

- Sensor fusion improved stability and positioning accuracy.

- Combined sensing reduced individual sensor limitations.

- IMU and barometer improved hover stability.

- Vision and distance sensing improved docking precision.

# 11. Signal Noise and Filtering Study

The effect of signal noise and vibration disturbances on sensor readings was studied.

Basic filtering concepts for UAV sensing systems were investigated.

## Observations

- IMU readings were sensitive to vibrations.

- Electrical noise affected sensor accuracy.

- Filtering improved signal stability.

- Noise reduction improved flight stabilization performance.

# 12. IMU Calibration Study

The importance of IMU calibration for accurate orientation sensing was studied.

The following factors were analyzed:

- Gyroscope drift

- Accelerometer bias

- Sensor offset errors

## Observations

- Uncalibrated sensors produced inaccurate orientation estimation.

- Gyroscope drift accumulated over time.

- Calibration improved flight stability and positioning accuracy.

- Accurate calibration improved docking alignment precision.

# 13. Docking Sensor Requirements

Sensor requirements for autonomous parent–child docking operations were analyzed.

The influence of:

- Relative positioning

- Distance estimation

- Orientation sensing

- Hover stabilization

was studied.

## Observations

- Close-range sensing is essential during docking.

- Orientation sensing improves alignment accuracy.

- Multiple sensor integration improves reliability.

- Hover stability depends strongly on IMU feedback.

- Vision-based sensing improves autonomous docking feasibility.

# 14. Comparative Sensor Analysis

| Sensor | Main Function | Advantages | Limitations |
| --- | --- | --- | --- |
| Ultrasonic | Distance sensing | Accurate short-range sensing | Limited range |
| IR Sensor | Proximity sensing | Fast response | Sensitive to lighting |
| IMU | Orientation sensing | Essential for stabilization | Drift errors |
| GPS | Position tracking | Long-range navigation | Low close-range precision |
| Barometer | Altitude sensing | Hover stabilization | Pressure fluctuation sensitivity |
| Magnetometer | Direction sensing | Heading estimation | Magnetic interference |
| Camera | Visual guidance | Accurate alignment | Lighting dependency |

# 15. Key Findings

The conducted sensor study identified that:

- IMU sensors are critical for flight stabilization.

- Ultrasonic and IR sensors are useful for close-range docking.

- GPS is effective for navigation but insufficient for precise docking.

- Vision sensing improves alignment accuracy.

- Sensor fusion significantly improves overall system reliability.

- Signal filtering improves sensing accuracy and flight stability.

- Proper sensor calibration is essential for reliable UAV operation.

# 16. Conclusion

The sensor systems relevant to the parent–child docking drone project were studied with emphasis on navigation, stabilization, altitude estimation, and docking assistance.

The conducted research demonstrated that:

- IMU sensors are essential for stable UAV operation.

- Distance sensors improve close-range docking capability.

- Vision sensing enhances autonomous alignment.

- Sensor fusion improves positioning accuracy and flight reliability.

- Signal filtering and calibration improve sensing accuracy.

- Integrated sensing systems are necessary for successful autonomous docking operations.

The study validated the importance of combining multiple sensing technologies for stable and reliable parent–child drone docking applications.